- Place bike in stand. Release rim brakes* and remove wheels

- Tighten removable rear mech hanger

- Check frame and fork condition

- Check brake calipers are secure & undamaged

- Check brake pad wear & condition (consider replacing if needed)

- Check Cranks

- Check Bottom Bracket:

  - Check BB turns smoothly and is free of play

  - If applicable, check lock-ring is secure

- Check Pedal condition

- Check Wheel Hubs (axles turn smoothly & without play)

- Check Spoke tension

- Check rim condition

- If puncture suspected: replace inner with known good item, check rim tape and check for sharps

- Replace wheels, return bike to the ground to tighten

- *Restore rim brakes

- Tyres:

  - Inflate

  - Check condition

- With the bike on the ground:

- Handlebars, levers and fittings:

  - Quill stem? Check height limit (lower if needed)

  - Check Stem & Bars are secure

  - Check h/bar fixtures (eg levers, bar plugs, light brackets)

- Seat and seat post:

  - Check height limit (lower if needed)

  - Check condition & that saddle and post are secure

- Check number of brake systems

- Check brake cables / hoses

- Check brake pads are secure. If rim brakes, check position & direction of pads

- Perform brake lever squeeze test:

  - Squeeze levers hard, check they do not bottom out against bars. Repeat 3x

  - Apply front brake, try to roll the bike forward

  - Apply rear brake, try to roll the bike backward

- Check wheels spin freely & without brake rub

- Check the headset:

  - Check steering is smooth

  - Check headset for play

- Lift bike into stand

- Check rear mech cannot contact r/wheel spokes

- Check gear cables

- Check gear indexing

- Check chain wear

- Check condition of sprockets and chain-rings

- Check pedal and rear reflectors

- Check accessories

- Complete job-sheet, photograph both sides + bike

- Sanitize bike, tools used & yourself